# KR260 Aurora + FIFO + Pattern Generator Setup Guide

## Architecture Overview

Pattern Generator → AXIS FIFO → Aurora TX → Internal Loopback → Aurora RX → RX FIFO → ILA

## Step 1 — Create RTL File

Create axis_pattern_generator.v in Vivado using Add Sources → Create File → Verilog.

## Step 2 — Add RTL Source

Flow Navigator → Add Sources → Add or Create Design Sources → Add Files → select axis_pattern_generator.v

## Step 3 — Package RTL as IP

Tools → Create and Package New IP → Package current project → Finish → Review and Package.

## Step 4 — AXI Interface Naming

Use AXI Stream names:
m_axis_tdata
m_axis_tvalid
m_axis_tready
m_axis_tlast

## Step 5 — Open Block Design

Open IP Integrator → Open/Create Block Design

## Step 6 — Add Required IPs

Add:
- axis_pattern_generator
- AXI4-Stream Data FIFO
- Aurora 64B/66B
- ILA
- Zynq MPSoC
- proc_sys_reset

## Step 7 — Connect Pattern Generator to FIFO

Pattern Generator M_AXIS → FIFO S_AXIS

## Step 8 — Connect FIFO to Aurora

FIFO M_AXIS → Aurora S_AXI_TX

## Step 9 — Enable Aurora Loopback

Aurora Configuration → Enable Near-End PMA Loopback

## Step 10 — Add RX FIFO

Add second AXI4-Stream FIFO for Aurora RX path.

## Step 11 — Connect Aurora RX

Aurora M_AXI_RX → RX FIFO S_AXIS

## Step 12 — Clock Connections

Enable pl_clk0 in Zynq MPSoC.
Use same clock initially for:
- Pattern Generator
- FIFO TX
- FIFO RX

## Step 13 — Reset Connections

Use proc_sys_reset/peripheral_aresetn connected to all modules.

## Step 14 — Add ILA Probes

Probe:
- tdata
- tvalid
- tready
- tlast
- channel_up

## Step 15 — Validate Design

Tools → Validate Design

## Step 16 — Generate Bitstream

Generate HDL Wrapper → Generate Bitstream

## Step 17 — Hardware Debug

Program KR260 and open Hardware Manager.
Expected:
- channel_up = 1
- tvalid = 1
- tready = 1
- tdata increments
- tlast every 16 cycles

## Recommended Debug Signals

Monitor:
- TVALID
- TREADY
- TLAST
- FIFO FULL
- FIFO EMPTY
- channel_up

## Basic AXI Stream Flow

Transfer occurs only when:
TVALID = 1 AND TREADY = 1

## Final Beginner Architecture

Pattern Generator → FIFO → Aurora TX → Internal Loopback → Aurora RX → FIFO → Checker → ILA